## CAVS REPORT

MSU.CAVS.CMD.2009-Rxxxx

# Printed month day, 2009

A user interface for the visco-plastic self-consistent (VPSC) model

VPSC7b_gui Users Manual

R. Cariño, H. ElKadiri, Q. Ma, and A. Oppedal

Prepared by

Center for Advanced Vehicular Systems

Mississippi State University

Mississippi State, MS 39762

Web site: http://www.cavs.msstate.edu

Not for public release.

Month 2009

Summary

This document describes VPSC7b_gui – a graphical user interface and calibration tool for the visco-plastic self-consistent (VPSC) model of plastic deformation of polycrystalline aggregates. The interface utilizes the model code, input files and output files described in Manual for Code Visco-Plastic Self-Consistent (VPSC), Version 7b-May 2007, by C.N. Tomé and R.A. Lebensohn, Los Alamos National Laboratory, USA. VPSC7b_gui attempts to calculate model parameters that minimize the difference between experimental stress-strain data and the stress-strain curve generated by VPSC. If fitting multiple data sets simultaneously, VPSC7b_gui exploits multi-processing capabilities of the computational environment to generate the model stress-strain curves in parallel.

Distribution List:

CAVS CMD students and faculty

C.N. Tomé, Los Alamos National Laboratory

R.A. Lebensohn, Los Alamos National Laboratory

Software Application License Agreement

Version 3.23.09b

This License Agreement (“Agreement”) is a legal agreement between LICENSEE and Mississippi State University (“MSU”) for online access to the software application for material modeling (“Software Application”). By using the Software Application, LICENSEE is agreeing to be bound by the terms of this Agreement. LICENSEE agrees that LICENSEE’s use of the Software Application acknowledges that LICENSEE has read this Agreement, understands it, and agrees to be bound by its terms and conditions. 

1. 	Grant of License. Subject to the terms and conditions of this Agreement, MSU hereby grants to LICENSEE a non-exclusive license to use the Software Application. LICENSEE may use the Software on a single computer and make one copy of the Software in machine-readable form for backup purposes only. LICENSEE must reproduce on such copy MSU’s copyright notice and any other proprietary legends that were on the original copy of the Software.

2. 	Restrictions. The Software Application contains copyrighted material, trade secrets and other proprietary material. In order to protect them, and except as permitted by applicable legislation, LICENSEE may not decompile, reverse engineer, disassemble, or otherwise reduce the Software Application to a human-perceivable form, or otherwise attempt to discover the source code of any Software Application provided in compiled form; modify, network, rent, lend, loan, distribute or create derivative works based upon the Software Application in whole or in part; electronically transmit the Software Application from one computer to another or over a network; copy the Software Application; or, transfer LICENSEE’s license rights to use the Software Application,  the related documentation, or this Agreement to another party.

3. 	Copyright and Trademark.  The Software Application and Documentation are owned by MSU. All title, ownership, rights, and intellectual property rights in and to the Software Application and Documentation shall remain with and in MSU. The Software Application and the Documentation are protected by the copyright laws of the United States and international copyright treaties. LICENSEE shall not use the names, logos, trademarks or any other mark or image considered by MSU to be identified with or protected by MSU, or those of any of its employees or former employees, or any adaptation thereof, in any advertising, promotional or sales literature without prior written consent being obtained from MSU in each case, except that LICENSEE may state that it is licensed by MSU under this Agreement.

4. 	Export Law Assurance.  LICENSEE hereby agrees that it shall not sell, transfer, export or re-export any Software Application or related information in any form, or any direct products of such information, except in compliance with all applicable laws, including the export laws of any U.S. Government agency and any regulations thereunder, and will not sell, transfer, export or re-export any such Software Application or information to any persons or any entities with regard to which there exist grounds to suspect or believe that they are violating such laws. LICENSEE shall be solely responsible for obtaining all licenses, permits or authorizations required from the U.S. and any other government for any such export or re-export.  To the extent not inconsistent with this Agreement, MSU agrees to provide LICENSEE with such assistance as it may reasonably request in obtaining such licenses, permits or authorizations.

5.	Disclaimer of Warranty.  MSU MAKES NO REPRESENTATIONS AND EXTENDS NO WARRANTIES OF ANY KIND, EITHER EXPRESS OR IMPLIED, INCLUDING, BUT NOT LIMITED, TO WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE, VALIDITY OF COPYRIGHT OR PATENT, ISSUED OR PENDING, OR FOR THE ABSENCE OF LATENT OR OTHER DEFECTS, WHETHER DISCOVERABLE OR NOT DISCOVERABLE.  NOTHING IN THIS LICENSE AGREEMENT SHALL BE CONSTRUED AS A REPRESENTATION MADE OR WARRANTY GIVEN BY MSU THAT THE PRACTICE BY LICENSEE OR SUBLICENSEES OF THE LICENSE GRANTED HEREUNDER SHALL NOT INFRINGE THE COPYRIGHT OR PATENT OF THIRD PARTIES.  IN NO EVENT SHALL THE MISSISSIPPI BOARD OF TRUSTEES OF STATE INSTITUTIONS OF HIGHER LEARNING, MSU THE TRUSTEES, OR ANY OFFICERS, AGENTS OR EMPLOYEES THEREOF BE LIABLE FOR INCIDENTAL OR CONSEQUENTIAL DAMAGES OF ANY KIND, INCLUDING ECONOMIC DAMAGE OR INJURY TO PROPERTY AND LOSS OF PROFITS, REGARDLESS OF WHETHER MSU SHALL BE ADVISED OF, SHALL OTHERWISE HAVE REASON TO KNOW, OR IN FACT SHALL KNOW OF THE POSSIBILITY THEREOF.

6.	MISCELLANEOUS PROVISIONS

A.	Governing Laws. This Agreement shall be construed, governed, interpreted and applied in accordance with the laws of the State of Mississippi, U.S.A without regard to its choice of law or conflicts of law rules or principles.

B.	Severability. The provisions of this Agreement are severable, and in the event that any provisions of this Agreement shall be determined to be invalid or unenforceable, such invalidity or unenforceability shall not in any way affect the validity or enforceability of the remaining provisions hereof.

C.	No Waiver. The failure of either party to assert a right hereunder or to insist upon compliance with any term or condition of this Agreement shall not constitute a waiver of that right or excuse a similar subsequent failure to perform any such term or condition by the other party.

D.	Lawful Use. LICENSEE shall not use Software Application for any unlawful purpose, including infringement of copyrights or proprietary rights of others, or in any illegal manner or for the creation or distribution of illegal content.

E.	Entire Agreement. The parties hereto acknowledge that this Agreement and any Appendices thereto, sets forth the entire Agreement and understanding of the parties hereto as to the subject matter hereof, and shall not be subject to any change or modification except by the execution of a written instrument subscribed to by the parties hereto.

## Table of Contents

0. For the impatient: A Quickstart Guide to VPSC7b_gui 	 7

1. Overview of the VPSC code	 10

2. The VPSC7b_gui application	 11

3. Files and directories 	 25

4. Example screenshots  (AM30) 	 27

5. Lessons learned  	 29

References 	 30

Appendix A. Source code	 31

Appendix B. Example properties+constraints file	 35

0. For the impatient: A Quickstart Guide to VPSC7b_gui

On the MSU-HPC2 filesystem, the VPSC7b_gui files are in the directory

/cavs/cmd/data1/common/Model_Fitting_Tools/VPSC7b/ver.1.0.

The executables VPSC7b_gui and vpsc7b.x were tested to work on 64-bit Linux desktops and servers (Javelin, Bazooka, and Apex). The executables VPSC7b_gui_win.exe and vpsc7b_win.exe were tested on Windows PCs running XP Professional.

I. Install the MATLAB Compiler Runtime library and set some environment variables (do this once only).

On LINUX desktops:

1. Copy the above directory, to say, <your_VPSC7b_gui_work_area>

2. Install the MATLAB Compiler Runtime library. The installer is in

/usr/local/matlab/toolbox/compiler/deploy/glnxa64/MCRInstaller.bin.

Specify, for example, /your_work_area/MCR as the installation directory. The installer will for take a few minutes.

3. Set environment variables. Add the following "export ..." lines to your .bashrc:

export LD_LIBRARY_PATH=$LD_LIBRARY_PATH:/usr/local/matlab/sys/os/glnxa64:/usr/local/matlab/bin/glnxa64:/usr/local/matlab/sys/java/jre/glnxa64/jre1.6.0/lib/amd64/native_threads:/usr/local/matlab/sys/java/jre/glnxa64/jre1.6.0/lib/amd64/server:/usr/local/matlab/sys/java/jre/glnxa64/jre1.6.0/lib/amd64

export XAPPLRESDIR=/usr/local/matlab/X11/app-defaults

On Windows PC

1. Copy the above directory, to say, C:\VPSC7b

2. Install the MATLAB Compiler Runtime library. If MATLAB R2008a is installed on your PC, the installer is in

C:\Program Files\MATLAB\R2008a\toolbox\compiler\deploy\win32\

MCRInstaller.exe

Otherwise, copy the installer (300+MB !) from /cavs/cmd/data1/common/Model_Fitting_Tools/ MCRInstaller.exe

You need to log in with administrative privileges to run the installer.  The installer will for take a few minutes.

II. Prepare the list of experimental data files.

The required information for an experiment is:

STRAIN-STRESS - stress-strain data file name

ORIENTATION - 1=a (Y), 2=b (X), 3=tt (Z)

LOADING_TYPE - 1=tension, 2=compression, 3=torsion, 4=channel die, ...

TEMP - temperature (K)

RATE - strain rate

VPSC7_IN - vpsc7.in

The experiment list looks like the following example:

STRAIN-STRESS= ED_AM30_25C_Neg.txt

ORIENTATION= 3

LOADING_TYPE= 2

TEMP= 298

RATE= 1e-3

VPSC7_IN= vpsc7.in

Add similar lines for additional experiments. The filenames may be absolute paths; otherwise, these as interpreted as relative to the directory containing the VPSC7b_gui executable.

III. Prepare the VPSC input files, one vpsc7.in per data set.

The input files are: vpsc7.in and all files (crystal files, texture files, etc) referenced by it.

IV. Run VPSC7b_gui.

Open a new terminal or command line window.

Change directory to <your_VPSC7b_gui_work_area>, and then type VPSC7b_gui/ VPSC7b_gui_win.exe on the command line to start the GUI. The first run will take a few seconds to initialize because the application files need to be extracted.

Select menu item File | Configure, then select the VPSC executable (vpsc7b.x for Linux, vpsc7b_win.exe for Windows), the host to run the executable (if Linux), and the working directory.

Select menu item File | Load data files, and then navigate to the list created in Step II.

Select menu item File | Initialize properties from crystal file to initialize the constants.

Click Evaluate push button to generate the model from the displayed properties. Depending on the model, the number of data files and the machine you selected, model generation may take several minutes.

V. Operations on data sets.

Select the data set to focus on using the DATA SET popup.

Click Exclude push button to temporarily exclude the data set from being plotted or fitted. Click Include push button for the opposite operation.

Click Add to add processes.

Click Del to delete the displayed process. You will not be allowed to delete all the processes for a data set.

Changes to data set elements (orientation, load type, nsteps, velocity grad., etc) and process element take effect after clicking the Apply changes push button.

VI. Operations on property sets.

Select the property set to focus on using the PROPERTY SET slider.

Click Remove to delete the property set on focus from memory.

Click MODES to activate/deactivate modes in a crystal file. An option to apply the selection to all crystal files will be presented as the last dialog.

Select a radio button to display the property values for that mode.

Right-click a property value to enter constraints for that property.

Click Apply changes to commit changes to property values. Several values from different modes may be changed before clicking this button. The next logical step would be to Evaluate the model for the updated property values.

VII. Files created by VPSC7b_gui.

The files created by VPSC7b_gui are located in a working directory specified during File | Configure. A subdirectory is created for each invocation of vpsc7b_lnx.x/vpsc7b_win.exe; this subdirectory contains the input files required by the VPSC program, the output files it creates, and a 'stdout.txt' which contains the console output.

Select menu item File | Write a binary restart file to make a record of the state of your session. To continue the session at a later time, select menu item File | Load a binary restart file

Select menu item File | Write properties+constraints file to create a text file copy of the current property set. This can be used as starting values in future fitting sessions by File | Load properties+constraints file

1. Overview of the VPSC code

From the VPSC manual [1]:

(page 2)

GENERAL DESCRIPTION

VPSC is a computer code written in FORTRAN 77 which simulates the plastic deformation of polycrystalline aggregates. VPSC stands for Visco Plastic Self Consistent and refers to the particular mechanical regime addressed (VP) and to the approach used (SC). VPSC was developed for application to low-symmetry materials (hexagonal, trigonal, orthorhombic, tetragonal), although it also performs well on cubic materials. VPSC accounts for full anisotropy in properties and response of the single crystals and the aggregate. It simulates the plastic deformation of aggregates subjected to external strains and stresses. VPSC is based on the physical shear mechanisms of slip and twinning, and accounts for grain interaction effects. In addition to providing the macroscopic stress-strain response, it accounts for hardening, reorientation and shape change of individual grains. As a consequence, it predicts the evolution of hardening and texture associated with plastic forming. The simulation procedure can be applied to deformation of metals, intermetallics and geologic aggregates.

…

(page 40)

2-4 CODE ARCHITECTURE

The modules of the code are: 1) a driver VPSC7.FOR; 2) an array declaration file VPSC7.DIM; 3) a library of specific plasticity routines VPSC7.SUB; 4) a library of general numerical routines LIBRARY7.SUB.

…

(page 44+)

2-6 DESCRIPTION OF INPUT FILES…

2-6-1 File VPSC7.IN …

2-6-2 PROCESS file (path & name read from VPSC7.IN)…

2-6-3 File FILETEXT (path & name read from VPSC7.IN) …

2-6-4 File FILECRYS (path & name read from VPSC7.IN) …

…

(page 55+)

2-7 DESCRIPTION OF OUTPUT FILES …

2-7-1 Log file RUN_LOG.OUT (unit=10) …

2-7-2 Optional output file PCYS.OUT (unit=14) …

2-7-3 Optional output file LANKFORD.OUT (unit=15) …

2-7-4 Convergence control file RERR.OUT (unit=12) …

2-7-5 Mode Activity Statistic file ACT_PHn.OUT (units=51,52, …, one per phase) …

2-7-6 Overall statistics file STATS.OUT (unit=11) …

2-7-7 Stress-strain file STR_STR.OUT (unit=13) …

2-7-8 Final state in grains and PX file POSTMORT.OUT (unit=19) …

2-7-9 Output texture files TEX_PHn.OUT or TEX_ELn.OUT (units=31, 32 ….) …

2-7-10 Final morphologic texture MOR_PHn.OUT or MOR_ELn.OUT (units=41,42 ….)…

2-7-11 Rolling components for each phase CUBCOMPn.OUT (units=61, 62 ….) …

2-7-12 Convergence control file for the second-order procedure SO.OUT (unit=97) …

2-7-13 Average fluctuations and SD per grain FLUCT.OUT (unit=83) …

As of version 7b, the VPSC code is still text-based and executed from a command line. One or more input files have to be manually edited before a computational deformation experiment can be started. The output files of interest have to be post-processed and transformed as inputs for other applications before the results can be visualized. In addition, calibrating the model parameters in FILECRYS to produce a STR_STR.OUT that will match an experimental stress-strain dataset is a very tedious, mechanical, and error-prone undertaking. Such was the experience of researchers in MSU-CAVS-CMD when they first utilized the VPSC to model deformation of magnesium, and thus, the motivation to create a user-friendly graphical user interface to VPSC that incorporates a model fitting functionality.

We developed the application VPSC7b_gui – a graphical user interface (GUI) to the VPSC executable. The development was guided by the following requirements: 1). VPSC7b_gui must be a “point and click” interface; 2). it must incorporate a model calibration functionality; and 3). revisions to the VPSC source code must be minimal, with no changes to the program logic. For 3), the only changes were: a) revise some of the I/O unit numbers, and b). add output statements so that the VPSC program will write ‘STOPPED’ to standard output just before it ends, for any reason.

An outline of the remainder of this report is as follows: Section 2 provides a snapshot of the interface and explains the function of each user interface control. Section 3 describes the plots, files and directories created by the interface. Section 4 includes screenshots form a sample fitting with AM30. Section 5 relates some of the lessons learned by the first users of the interface. Finally, the Appendix lists the source code.

2. The VPSC7b_gui application

User interaction with the VPSC code typically proceeds as follows:

- Prepare the input files (VPSC7.IN, PROCESS file, file FILETEXT, and file FILECRYS;

- Execute the VPSC binary; and

- Generate visualization plots from the relevant output files using other software.

When fitting the Voce parameters and the latent hardening coupling parameters in FILECRYS to experimental stress-strain data, the user takes the following additional steps:

- Estimate the error of the model curve in STR_STR.OUT from the experimental data;

- If the error estimate is still large, modify the relevant parameters in FILECRYS;

- Repeat from Step 2 until the error estimate becomes acceptable.

The above procedure is very tedious and time-consuming when done manually, especially if calibrating with multiple stress-strain datasets representing different loading conditions. Hence, we developed the VPSC7b_gui application as an alternative graphical semi-automatic interface to VPSC  implementing the above procedure. The application assumes that a VPSC7.IN ‘master’ input file is associated with each experimental stress-strain dataset utilized for calibration. Multiple datasets are admissible, each dataset possibly having its own PROCESS file, FILETEXT and FILECRYS, provided that the same slip and twin deformation modes are specified for all datasets.

MATLAB [2] (version R2008a) was chosen as the platform for the interface since MATLAB includes functionalities for GUI development, system calls, and visualization, in a single package. In addition, compiled MATLAB applications are redistributable royalty-free.

Figure 2 is a snapshot of the VPSC7b_gui application window. The remainder of this section explains the function of each user interface control element.

Figure 2. A snapshot of the VPSC7b_gui window.

2.1 Menu items

2.1.1 File menu items

File | Configure VPSC. Open the dialogs to specify the name VPSC binary, the root of the working directory where files will be stored by VPSC7b_gui, and the name of the host to execute the VPSC binary. By default, VPSC7b_gui and VPSC will run on the same machine. If another host will run VPSC, the (local) machine running VPSC7b_gui must be able to rsh to the (remote) host which will run VPSC, and the working directory must be accessible to both the local machine and the remote execution host. For example, VPSC7b_gui may be running on a user’s desktop, while the machine to run the VPSC executable maybe a multiprocessor compute server.  The working directory may be on the user’s home which is on a filesystem that is visible to the local and remote machines.

The functionality to specify the VPSC executable allows the user to experiment with variants of the release version of the code, for example, to test revisions to the underlying formulas. When calibrating the model using multiple data sets, specifying a multiprocessor execution host for VPSC enables VPSC7b_gui to generate the model stress-strain curves for the datasets in parallel.

File | Load data files. Open the dialog to select a text file that lists the experimental stress-strain datasets and their associated VPSC7.IN master input files. A template for an experiment list is the following:

#

# List of experimental data files and their corresponding vpsc7.in

# Lines starting with ‘#’ are comments

#

# STRAIN-STRESS: stress-strain data file name

# ORIENTATION  : 1=a (Y), 2=b (X), 3=tt (Z)

# LOADING_TYPE : 1=tension, 2=compression, 3=torsion, 4=channel die, ...

# TEMP         : temperature (K)

# RATE         : strain rate

# VPSC7_IN     : path to VPSC7.IN for the data file

STRAIN-STRESS= DOE/AM30/ED_AM30_25C_Neg/ED_AM30_25C_Neg.txt

ORIENTATION= 3

LOADING_TYPE= 2

TEMP= 298

RATE= 1e-3

VPSC7_IN= DOE/AM30/ED_AM30_25C_Neg/vpsc7.in

STRAIN-STRESS= DOE/AM30/ET_AM30_25C_Neg/ET_AM30_25C_Neg.txt

ORIENTATION= 1

LOADING_TYPE= 2

TEMP= 298

RATE= 1e-3

VPSC7_IN= DOE/AM30/ET_AM30_25C_Neg/vpsc7.in

At least one experiment must be listed. The filenames for stress-strain dataset and its VPSC7.IN master input file may be absolute filenames, or relative to the directory containing the VPSC7b_gui application executable like in the above template.

File | Initialize properties from crystal file. The Voce parameters and latent hardening parameters are all set to 0 when VPSC7b_gui starts. This menu item opens a dialog to select, from among the crystal files listed in all the VPSC7.IN files, a FILECRYS from which to initialize the Voce parameters and the latent hardening parameters.

File | Load properties+constraints file. Open a dialog to select a “properties and constraints” (*.props) file. This is a text file containing lines with the format "name=value fix min max", where "name" is a fitted constant in the VPSC model, fix is 0 or 1 to indicate that the value is to be fixed during optimization, and [min, max] is the range of permissible values for the constant if fix=0. See Appendix B for an example .props file. A .props file may be created by File | Write properties+constraints file.

File | Load binary restart file. A binary restart file is a record of the state of VPSC7b_gui during a previous fitting session, created by File | Write binary restart file during that session. This menu item opens a dialog to select the restart file for the fitting session to be resumed.

File | Retrieve a previous run. The VPSC executable is invoked by VPSC7b_GUI when the Evaluate pushbutton or the Optimize pushbutton is selected. Each invocation of VPSC creates a ‘run’ directory. The structure of the directory and its contents are described in Section 3. This menu item opens a dialog to select such a run directory and then display the results of that VPSC invocation without actually re-running the VPSC executable. For this menu item to function properly, the experiment list used by the previous VPSC invocation (see File | Load data files) must be the list that is currently loaded.

File | Write binary restart file. Open a dialog for the name of a *VPSC7b_gui.mat restart file, then create the file. The file records the state of the current fitting session. Loading this file in the future with Load | Binary restart file continues the session. This feature is useful if the user gets tired and wishes to continue the fitting session after getting some sleep.

File | Write properties+constraints file. Open a dialog for the name of a .props file, then write the file. This is a text file containing the current values and constraints for the Voce parameters and the latent hardening parameters. This file may be used as input (see File | Load properties+ constraints file) in future fitting sessions. See Appendix B for an example .props file.

File | Write Voce hardening plot values. Open a dialog for the name of a .csv file, then write the file. This is a text file containing the Voce hardening plot values for post-processing. This file can be imported as comma-separated-values (CSV) into a spreadsheet program like MS Excel.

File | Print figure. Open a print preview of the VPSC7b_gui window. A snapshot of the window can then be sent directly to a printer.

File | Reset. Re-initialize VPSC7b_gui.

File | Quit. Quit VPSC7b_gui.

2.1.2 Optimization menu items

Optimization | Use fminsearch. Set options for MATLAB's fminsearch(), then use it as the optimization routine.

Optimization | Use fmincon. Set options for MATLAB's fmincon(), then use it as the optimization routine.

Optimization | Use lsqnonlin. Set options for MATLAB's lsqnonlin(), then use it as the optimization routine.

Optimization | Use pattern search. Set options for MATLAB's patternsearch(), then use it as the optimization routine.

Optimization | Use genetic algorithm. Set options for MATLAB's ga(), then use it as the optimization routine.

2.1.3 Plot menu items

Plot | Plot settings. Open a dialog to modify the appearance of the plot area.

Plot | Hide|Show STR_STR plots. Toggle the switch to hide or show model stress-strain plots.

Plot | Hide|Show Voce hardening plots. Toggle the switch to hide or show Voce hardening plots.

Plot | Hide|Show ACT, PCYS, Lankford plots. Toggle the switch to hide or show activity plots, polycrystal yield surface plots, or Lankford plots.

2.1.4 Help menu items

Help | Topics. Display the help file.

Help | About. Display the current version of VPSC7b_gui and the contact information of the developer/maintainer.

2.2 Data set controls

In VPSC7b_gui, a data set is specified by the following elements in an experiment list (see File | Load data files):

- STRAIN-STRESS – path to file containing the experimental stress-strain data. This text file must contain two columns of numbers: first column gives the strain and the second column gives the stress (MPa). For compression data, the numbers will be negative.

- ORIENTATION – the load direction, used to determine which columns in STR_STR.OUT will be extracted as the model for the experimental data: 1=In-plane longitudinal, use (E22, SCAU22); 2=In-plane transverse, use (E11, SCAU11); 3=Through thickness, use (E33, SCAU33).

- LOADING_TYPE – the experiment type: 1=tension, 2=compression, 3=torsion, 4=channel die.

- TEMP – the temperature (K).

- RATE – the strain rate (1/sec).

- VPSC7_IN – the path to the master input file.

The user interface controls for data set operations are:

DATA SET popup menu – select the data set whose elements will be displayed. If none is selected, the rest of the data set controls will be filled with blanks; otherwise, the controls will be updated.

orient edit box – the ORIENTATION for the data set.

load type edit box – the LOADING_TYPE.

rate edit box – the RATE.

Excl push button – temporarily exclude the selected data set from being plotted and from participating in model calibration. If an excluded data set is selected, this push button will change to Incl.

Process popup menu – select a process listed in the VPSC7.IN of the data set. The other process controls will be updated depending on the IVGVAR for the selected process.

Del push button – delete the selected process. The user is not allowed to delete all processes; at least one will remain. To delete a solitary process, the replacement process must first be added (see Add push button).

Add push button – open a series of dialogs to add a process. The dialogs depends on the specified IVGVAR: if IVGVAR=0 or 1, the next dialog will establish FILEHIST; if IVGVAR=2, the succeeding dialog will establish the components of the projection subspace; if IVGVAR=3, the next dialog will establish the angular increment (in degrees); if IVGVAR=4, the next dialog will select the name of the file containing the 3x3 rotation matrix.

IVGVAR text box – the IVGVAR for the selected process.

D.LANK text box – the angular increment if IVGVAR=3.

INDX, INDY text boxes – the first and second components of the projection subspace if IVGVAR=2.

FILEHIST text box – the name of the PROCESS file if IVGVAR=0, or the file containing the 3x3 rotation matrix if IVGVAR=4.

nsteps, ictrl, ctrlincr, temp edit boxes – the values in the first line of the PROCESS file if IVGVAR=0.

Velocity gradient edit boxes, flag check boxes – UDOT and IUDOT in the PROCESS file if IVGVAR=0.

Cauchy stress edit boxes, flag check boxes – SCAUCHY and ISCAU in the PROCESS file if IVGVAR=0.

Changes to the above edit boxes and check boxes take effect after selecting the Apply Changes push button. A warning similar to the one below may be issued as a consequence of the changes.

2.3 Plot areas

VPSC7b_gui produces a number of plots depending on the number of data sets, the processes for each data set, and the Plot menu settings. The active data sets (not Excluded) and their corresponding models (if available in STR_STR.OUT) are always plotted in the main display area, like in Figure 2. In addition, there may be plots produced from ACT_PHn.OUT, PCYS.OUT and LANKFORD.OUT. These plots will be detached from the VPSC7b_gui window. See Figure 3 for screenshots.

Figure 3. Other plots produced by VPSC7b_gui.

2.4 Property set controls

In VPSC7b_gui, a property refers to a Voce hardening parameter or a latent hardening parameter in the VPSC model. In the user interface, these are designated, for each mode, as tau0, tau1, theta0, theta1, HP, GND, and HLATEX1, HLATEX2, …, HLATEX7. The active modes are represented by enabled radio buttons, e.g., Prismatic, Basal, Pyramidal2 <c+a>, and Twin1, for example in Figure 4 below. The currently displayed values are those of the selected radio button, e.g., Prismatic. The parameters for another active mode may be displayed by selecting the radio button for the mode.

Figure 4. The user interface controls for property sets and properties.

The MODES push button opens a series of dialogs to activate or deactivate deformation modes. The first dialog selects which FILECRYS file to modify, the second dialog establishes which modes will be active and inactive, and the third dialog presents an option to apply the modifications to all FILECRYS files.

A property is associated with four interface elements: the current value edit box, a range of permissible values during optimization (not visible), and a fix check box to indicate that the property is fixed (not to be changed) during optimization. The range is displayed by hovering the pointing device over the current value edit box (see light blue box in Figure 4).

A property set is simply the collection of all the properties. VPSC7b_gui maintains a record of all property sets attempted. A user navigates this record by moving the PROPERTY SET slider. In Figure 4, the slider focuses on property set #2 of the two sets available. The property set on focus is discarded by selecting the Remove push button, after which the next available parameter set automatically becomes the focus.

A dialog to modify the values associated with a property is initiated with a right-click on the current value edit box. For example, the dialog on the right figure appears after a right-click on tau0 edit box. After changing any of the values, selecting the OK pushbutton makes a temporary copy of the new property. These changes are committed only after selecting the Apply changes push button, which creates a new property set by merging the modified property into the property set on focus.

Several properties may be modified before selecting the Apply changes push button to create a new property set. A VPSC model is not automatically available for this new set; thus, model plots for the previous property set on focus, if any, are erased when selecting the Apply changes push button.

The Evaluate push button creates an ‘evaluate directory’ for each active data set, invokes the VPSC executable, waits for completion, then plots the models from the STR_STR.OUT. The directory contains all the input required by the VPSC code, as well as the output files it creates. The directory structure is described in Section 3.

The Optimize push button executes the VPSC model calibration process illustrated in Figure 5. The inputs are the initial values for the Voce parameters and the latent hardening parameters of the current property set, and the experimental stress-strain data sets. An optimization routine (selected by the menu item Optimization) generates candidate constants and writes a new FILECRYS for each data set. The VPSC executable is invoked with the modified FILECRYS to produce a model stress-strain curve. The model curve is compared with the experimental data by the optimizer, to generate the next set of candidate constants. The candidate constants are written to the FILECRYS for each data set, and the cycle is repeated. Successful termination occurs when the model curves give good correlations to the experimental data.

The calibration process may take several hours or days if not manually terminated. The models for candidate property sets attempted during optimization are plotted for monitoring progress. Input files required for each attempt and the output files created are stored in an ‘optimize directory’ described in Section 3. When using more than one data set for calibration, VPSC7b_gui takes advantage of multiprocessing capabilities of the execution host to reduce computation time by making concurrent invocations of the VPSC executable.

Figure 5. Overview of a semi-automated VPSC model calibration process.

Section 3. Files and directories

The VPSC7b_gui working directory is specified during configuration (File | Configure). This directory stores all files created by VPSC7b_gui as inputs for the VPSC executable, as well as the output files created by VPSC. The directory is organized as illustrated by Figure 6 below.

VPSC7b_gui_working_directory

|

|-- timestamp_evaluate

|           |

|           |-- dataset1

|           |      |

|           |      |- STR_STR.OUT, ACT_PH1.OUT, TEX-PH1.OUT, ...

|           |

|           |-- dataset2

|                  |

|                  |- STR_STR.OUT, ACT_PH1.OUT, TEX-PH1.OUT, ...

|

|-- timestamp_optimize

|           |

|           |-- fn.eval 1

|           |      |

|           |      |-- dataset1

|           |      |      |

|           |      |      |- STR_STR.OUT, ACT_PH1.OUT, TEX-PH1.OUT, ...

|           |      |

|           |      |-- dataset2

|           |             |

|           |             |- STR_STR.OUT, ACT_PH1.OUT, TEX-PH1.OUT, ...

|           |

|           |-- fn.eval 2

|           |      |

|           |      |-- dataset1

|           |      |      |

|           |      |      |- STR_STR.OUT, ACT_PH1.OUT, TEX-PH1.OUT, ...

|           |      |

|           |      |-- dataset2

|           |             |

|           |             |- STR_STR.OUT, ACT_PH1.OUT, TEX-PH1.OUT, ...

Figure 6. Organization of the VPSC7b_gui working directory.

When the Evaluate push button is selected, VPSC7b_gui creates the subdirectory timestamp_evaluate, where timestamp is  yyyymmddTHHMMSS (the current year, month, day, ‘T’, hours, minutes, and seconds in the machine executing VPSC7b_gui). VPSC7b_gui also creates, under timestamp_evaluate, subdirectories for each data set, named with the data set filenames. VPSC7b_gui writes the VPSC7.IN master input file and the files referred by it (FILECRYS, FILEHIST, FILETEXT, etc.) to each data set subdirectory. VPSC7b_gui then invokes the VPSC executable from each data set subdirectory, to create the output files (RUNLOG.OUT, RERR.OUT, STR_STR.OUT, ACT_PHn.OUT, etc.). The VPSC model code must be modified to write ‘STOPPED’ to standard output whether it completes successfully or not. VPSC7b_gui redirects the standard output of VPSC to the file ‘stdout.txt’ in the data set subdirectory. VPSC7b_gui examines this file for a line containing the text ‘STOPPED’ as an indication that the VPSC executable has completed. For successful completions, VPSC7b_gui will update the plots from STR_STR.OUT, ACT_PHn.OUT, etc).

When the Optimize push button is selected, VPSC7b_gui creates the subdirectory timestamp_optimize, where timestamp is as above. The optimization routine attempts to minimize an objective function representing the difference between the VPSC model and the experimental data. To do this, the optimization routine evaluates the objective function a few hundred or thousand times. Each evaluation requires an invocation of the VPSC executable; thus, VPSC7b_gui creates a subdirectory similar to the timestamp_evaluate subdirectory, named after the function evaluation count (fn.eval in Figure 6).

The above organization makes it easy to differentiate VPSC files used for simple evaluations from the files used for optimizations. Further, naming the optimization directories with the function evaluation counts provides a means of ‘indexing’ the optimization iteration steps, making it easy to determine where to find property sets for restarting the optimization.
Section 4. Example screenshots (AM30)

The example described in this section pertains to AM30. Simple compression tests were performed on pre-extruded AM30 material parallel and normal to the extrusion direction at 298K temperature and 10-3/s strain rate. The sample having compression axis coinciding with extrusion direction is designated by ED (Extrusion Direction). The sample having compression axis normal to the extrusion direction is referred to by ET (Extrusion Transverse).  Figure 7 illustrates the origin of the two samples. Figure 8 provides a schematic of the slip and twinning modes considred in the model. Figure 9 plots the experimental stress-strain and the calibrated VPSC model. Figures 10 – 12 depict the other quatities of interest that are generated by VPSC.

Figure 7. Schematic illustration of the location and orientation of the samples.

Figure 8. Schematic illustration of the slip and twinning modes for compressed AM30.

Figure 8. Experimental stress-strain data (discrete points) and VPSC model (solid line).

| Figure 9. Relative activity of various deformation modes along strain. | Figure 10. Twin volume fraction and effective twin volume fraction along strain. |
| --- | --- |
| Figure 11. Stress-strain of various deformation modes along accumulated shear strain. |  |

Section 5. Lessons learned

Determining the proper Voce parameters for a material is a trial and error process, and can be time consuming and tedious. Starting with reasonable guesses for initial values, perhaps from a similar material, many runs of the simulation code will be required to arrive at a solution. The best approach is to guide the approach with some thoughtful consideration.

In the absence of the GUI discussed here, the changes for each simulation are accomplished by editing the input files such as the crystal files with the Voce parameters. Then the VPSC simulations are run from a command prompt, and the prediction is compared to the experimental results. Changes are made to the parameters, and then the simulations are run again. In the beginning of the process, comparison of the mechanical stress-strain response is most important. As the parameters are refined, the activity files of the individual deformation modes must also be consulted. As a final check, the texture files are also compared.

The first step is to consider the deformation modes active in the material. This can be done via a review of the literature. The literature is not always consistent, however. The initial yield  should be determined and is independent of the hardening parameters. After the initial yield, then the initial hardening slope, , should be determined. The saturation stress, , can be determined next. The determination of latent hardening parameters is more difficult, and should be done with experimental results.

The GUI discussed here is helpful to make changes in the parameters and see the effect on the response. The optimization routines are available, but should not be relied on. Optimization will produce a mathematically optimal solution, but this may not correspond to the underlying physics. The better use of the optimization is to use guided, human direction of the process, and use optimization on just a few parameters at any one time to improve the solution. Using the optimization feature to produce a solution, allowing the procedure to change many variables at once, will almost certainly lead to a lengthy process without good results.

References

[1] Manual for Code Visco-Plastic Self-Consistent  (VPSC), Version 7b-May 2007, by C.N. Tomé and R.A. Lebensohn, Los Alamos National Laboratory, USA.

[2] MATLAB, http://www.matworks.com/matlab

Appendix A. Source code

function varargout = VPSC7b_gui(varargin)

% VPSC7b_gui M-file for VPSC7b_gui.fig

%      VPSC7b_gui, by itself, creates a new VPSC7b_gui or raises the existing

%      singleton*.

%

%      H = VPSC7b_gui returns the handle to a new VPSC7b_gui or the handle to

%      the existing singleton*.

%

%      VPSC7b_gui('CALLBACK',hObject,eventData,handles,...) calls the local

%      function named CALLBACK in VPSC7b_gui.M with the given input arguments.

%

%      VPSC7b_gui('Property','Value',...) creates a new VPSC7b_gui or raises the

%      existing singleton*.  Starting from the left, property value pairs are

%      applied to the GUI before VPSC7b_gui_OpeningFcn gets called.  An

%      unrecognized property name or invalid value makes property application

%      stop.  All inputs are passed to VPSC7b_gui_OpeningFcn via varargin.

%

%      *See GUI Options on GUIDE's Tools menu.  Choose "GUI allows only one

%      instance to run (singleton)".

%

% See also: GUIDE, GUIDATA, GUIHANDLES

% Edit the above text to modify the response to help VPSC7b_gui

% Last Modified by GUIDE v2.5 03-Apr-2009 08:38:10

% Begin initialization code - DO NOT EDIT

gui_Singleton = 1;

gui_State = struct('gui_Name',       mfilename, ...

'gui_Singleton',  gui_Singleton, ...

'gui_OpeningFcn', @VPSC7b_gui_OpeningFcn, ...

'gui_OutputFcn',  @VPSC7b_gui_OutputFcn, ...

'gui_LayoutFcn',  [] , ...

'gui_Callback',   []);

if nargin && ischar(varargin{1})

gui_State.gui_Callback = str2func(varargin{1});

end

if nargout

[varargout{1:nargout}] = gui_mainfcn(gui_State, varargin{:});

else

gui_mainfcn(gui_State, varargin{:});

end

% End initialization code - DO NOT EDIT

% --- Executes just before VPSC7b_gui is made visible.

function VPSC7b_gui_OpeningFcn(hObject, eventdata, handles, varargin)

% This function has no output args, see OutputFcn.

% hObject    handle to figure

% eventdata  reserved - to be defined in a future version of MATLAB

% handles    structure with handles and user data (see GUIDATA)

% varargin   command line arguments to VPSC7b_gui (see VARARGIN)

% Choose default command line output for VPSC7b_gui

handles.output = hObject;

handles.is_configured = false;

% set(handles.progress, 'Visible', 'Off')

axes(handles.progress)

axis off

% Initialize user_data

handles.user_data = Initialize_UserData;

% Initilize Activity, PCYS, Lankford figures

% assume at most 5 ACT_PHk.OUT

handles.max_ACT = 5;

for IPH=1:handles.max_ACT

handles.MODE_Activity_axes{IPH} = [];

handles.TWIN_Activity_axes{IPH} = [];

end

handles.PCYS_S2vsS1_axes = [];

handles.PCYS_D2vsD1_axes = [];

handles.LANKvsANGLE_axes = [];

handles.YOUNGvsANGLE_axes = [];

handles.ACCUM_SHEAR_STRAIN_axes = [];

% Initialize GUI

handles = Initialize_GUI(handles, handles.user_data);

% Update handles structure & plots

guidata(hObject, handles)

% UIWAIT makes VPSC7b_gui wait for user response (see UIRESUME)

% uiwait(handles.VPSC7b_gui);

% --- Outputs from this function are returned to the command line.

function varargout = VPSC7b_gui_OutputFcn(hObject, eventdata, handles)

% varargout  cell array for returning output args (see VARARGOUT);

% hObject    handle to figure

% eventdata  reserved - to be defined in a future version of MATLAB

% handles    structure with handles and user data (see GUIDATA)

% Get default command line output from handles structure

varargout{1} = handles.output;

function VPSC_CloseRequestFcn(hObject, eventdata, handles)

% clean up & close figure

% delete(hObject)

File_Quit_Callback(hObject, eventdata, handles)

function Help_Callback(hObject, eventdata, handles)

% --------------------------------------------------------------------

function Help_Topics_Callback(hObject, eventdata, handles)

% hObject    handle to Help_Topics (see GCBO)

% eventdata  reserved - to be defined in a future version of MATLAB

% handles    structure with handles and user data (see GUIDATA)

% executes on click of help menu item

if ispc && exist ([mfilename '.chm'], 'file') % Windows w/ Compiled HTML help

winopen([mfilename '.chm'])

elseif exist ([mfilename '.htm'], 'file') % raw HTML help exists?

web(['file:///' pwd '/' mfilename '.htm'], '-browser')

else

msgbox('Help file not found!', 'Help', 'error', 'modal')

end

% --------------------------------------------------------------------

function Help_About_Callback(hObject, eventdata, handles)

% hObject    handle to Help_About (see GCBO)

% eventdata  reserved - to be defined in a future version of MATLAB

% handles    structure with handles and user data (see GUIDATA)

% Help_About

msgbox({get(handles.VPSC, 'Name'); ...

'by MSU-CAVS-CMD'; ' '; 'Send suggestions & bug reports to:'; ...

'RLC@CAVS.MsState.Edu'}, ...

'About', 'help', 'modal')

% --------------------------------------------------------------------

function Input_Datasets_Callback(hObject, eventdata, handles)

% hObject    handle to Input_Datasets (see GCBO)

% eventdata  reserved - to be defined in a future version of MATLAB

% handles    structure with handles and user data (see GUIDATA)

% load stress-strain data sets

if ~handles.is_configured

msgbox('Not configured? The VPSC executable, the machine to run VPSC, and the working directory must all  be specified.', '', 'Error', 'modal')

return

end

user_data = handles.user_data;

control_tag = get(hObject, 'Tag'); % GUI object tag

dialog_title = strrep(control_tag, '_', ' '); % use tag w/o "_" as popup window title

% get filenames

[data_file file_path] = uigetfile('vpsc7b_*.list*', 'Select experiment list', ...

user_data.last_dir_load, 'MultiSelect', 'Off');

if isequal(data_file, 0), return, end

data_file = fullfile(file_path, data_file);

% retrieve

[fid,msg] = fopen(data_file, 'r');

if fid == -1

msgbox(['Cannot read from ' data_file ' due to error "' msg '"'], dialog_title, 'error', 'modal')

return

end

h = msgbox(['Reading files listed in ' data_file ' may take a few seconds. Please wait ...'], ...

dialog_title, 'warn', 'modal');

% STRAIN-STRESS= exp_stress_strain/mg_tt_373_1e-2_9_sample.TXT

% TYPE= 3

% TEMP= 373

% RATE= 1e-2

% VPSC7_IN= mod_load_path/tt_373_e-2.txt

%

user_data.dataset = {};

nfiles = 0;

while feof(fid)==0

tline = fgetl(fid);

if isempty(tline) || strncmp(tline, '#', 1) % empty/comment line

continue

end

token = Tokenize(tline, '=');

switch token{1}

case 'ORIENTATION'

user_data.dataset{nfiles}.orientation = str2double(token{2});

case 'LOADING_TYPE'

user_data.dataset{nfiles}.loadtype = str2double(token{2});

case 'TEMP'

user_data.dataset{nfiles}.temp = str2double(token{2});

case 'RATE'

user_data.dataset{nfiles}.rate = str2double(token{2});

case 'STRAIN-STRESS'

nfiles = nfiles + 1;

user_data.dataset{nfiles} = user_data.dataset_default;

[fid1,msg] = fopen(token{2}, 'r');

if fid1 == -1

fclose(fid);

delete(h);

msgbox(['Cannot read from ' token{2} ' due to error "' msg '"'], ...

dialog_title, 'error', 'modal')

return

else

tvals = textscan(fid1, '%n %n');

fclose(fid1);

user_data.dataset{nfiles}.strain = tvals{1};

user_data.dataset{nfiles}.stress = tvals{2};

end

[t_path t_fname t_ext] = fileparts(token{2});

user_data.dataset{nfiles}.fname = t_fname;

case 'VPSC7_IN'

[vpsc7_in error_code error_text] = Input_vpsc7_in(token{2}, user_data);

if error_code

fclose(fid);

delete(h);

msgbox(['Error in ' token{2} ': ' error_text], dialog_title, 'error', 'modal')

return

end

user_data.dataset{nfiles}.vpsc7_in = vpsc7_in;

end

end

fclose(fid);

delete(h);

if nfiles==0

msgbox([data_file ' is not a valid list?'], dialog_title, 'error', 'modal')

else

% disable inactive modes

set(findobj('-regexp', 'Tag', 'mode_'), 'Enable', 'Off')

for i=1:nfiles

for IPH=1:user_data.dataset{i}.vpsc7_in.NPH

for j=i:user_data.dataset{i}.vpsc7_in.phase(IPH).NMODES

mode=user_data.dataset{i}.vpsc7_in.phase(IPH).MODE(j);

t_name = ['mode_' user_data.mode_label{mode}];

set(handles.(t_name), 'Enable', 'On')

end

end

end

% update handles & display

user_data.plot_option.title = 'Data plots';

handles.user_data = user_data;

guidata(hObject, handles)

Update_GUI(handles, user_data, [1 1 1 1])

end

For complete source code, please send a request to:

Ricolindo L Carino, RLC@CAVS.MsState.Edu

Appendix B. Example properties+constraints file

#

# Generated 31-Jul-2009 15:07:32

# Constants and constraints for Visco-plastic self-consistent 1-site multiphase (ver. 7b)

#

model_tag= VPSCgui

#

# Descriptions

#

# PRISMATIC_TAU0X - Initial CRSS

# PRISMATIC_TAU1X - Back propagated CRSS - TAU0X

# PRISMATIC_THET0X - Initial hardening rate

# PRISMATIC_THET1X - Asymptotic hardening rate

# PRISMATIC_HPFACX - Hall-Petch contribution

# PRISMATIC_GNDFACX - GND hardening term

# PRISMATIC_HLATEX1 - Latent hardening coupling coefficient - PRISMATIC

# PRISMATIC_HLATEX2 - Latent hardening coupling coefficient - BASAL

# PRISMATIC_HLATEX3 - Latent hardening coupling coefficient - PYRAMIDAL_A

# PRISMATIC_HLATEX4 - Latent hardening coupling coefficient - PYRAMIDAL_CA1

# PRISMATIC_HLATEX5 - Latent hardening coupling coefficient - PYRAMIDAL_CA2

# PRISMATIC_HLATEX6 - Latent hardening coupling coefficient - TWIN1

# PRISMATIC_HLATEX7 - Latent hardening coupling coefficient - TWIN2

# BASAL_TAU0X - Initial CRSS

# BASAL_TAU1X - Back propagated CRSS - TAU0X

# BASAL_THET0X - Initial hardening rate

# BASAL_THET1X - Asymptotic hardening rate

# BASAL_HPFACX - Hall-Petch contribution

# BASAL_GNDFACX - GND hardening term

# BASAL_HLATEX1 - Latent hardening coupling coefficient - PRISMATIC

# BASAL_HLATEX2 - Latent hardening coupling coefficient - BASAL

# BASAL_HLATEX3 - Latent hardening coupling coefficient - PYRAMIDAL_A

# BASAL_HLATEX4 - Latent hardening coupling coefficient - PYRAMIDAL_CA1

# BASAL_HLATEX5 - Latent hardening coupling coefficient - PYRAMIDAL_CA2

# BASAL_HLATEX6 - Latent hardening coupling coefficient - TWIN1

# BASAL_HLATEX7 - Latent hardening coupling coefficient - TWIN2

# PYRAMIDAL_A_TAU0X - Initial CRSS

# PYRAMIDAL_A_TAU1X - Back propagated CRSS - TAU0X

# PYRAMIDAL_A_THET0X - Initial hardening rate

# PYRAMIDAL_A_THET1X - Asymptotic hardening rate

# PYRAMIDAL_A_HPFACX - Hall-Petch contribution

# PYRAMIDAL_A_GNDFACX - GND hardening term

# PYRAMIDAL_A_HLATEX1 - Latent hardening coupling coefficient - PRISMATIC

# PYRAMIDAL_A_HLATEX2 - Latent hardening coupling coefficient - BASAL

# PYRAMIDAL_A_HLATEX3 - Latent hardening coupling coefficient - PYRAMIDAL_A

# PYRAMIDAL_A_HLATEX4 - Latent hardening coupling coefficient - PYRAMIDAL_CA1

# PYRAMIDAL_A_HLATEX5 - Latent hardening coupling coefficient - PYRAMIDAL_CA2

# PYRAMIDAL_A_HLATEX6 - Latent hardening coupling coefficient - TWIN1

# PYRAMIDAL_A_HLATEX7 - Latent hardening coupling coefficient - TWIN2

# PYRAMIDAL_CA1_TAU0X - Initial CRSS

# PYRAMIDAL_CA1_TAU1X - Back propagated CRSS - TAU0X

# PYRAMIDAL_CA1_THET0X - Initial hardening rate

# PYRAMIDAL_CA1_THET1X - Asymptotic hardening rate

# PYRAMIDAL_CA1_HPFACX - Hall-Petch contribution

# PYRAMIDAL_CA1_GNDFACX - GND hardening term

# PYRAMIDAL_CA1_HLATEX1 - Latent hardening coupling coefficient - PRISMATIC

# PYRAMIDAL_CA1_HLATEX2 - Latent hardening coupling coefficient - BASAL

# PYRAMIDAL_CA1_HLATEX3 - Latent hardening coupling coefficient - PYRAMIDAL_A

# PYRAMIDAL_CA1_HLATEX4 - Latent hardening coupling coefficient - PYRAMIDAL_CA1

# PYRAMIDAL_CA1_HLATEX5 - Latent hardening coupling coefficient - PYRAMIDAL_CA2

# PYRAMIDAL_CA1_HLATEX6 - Latent hardening coupling coefficient - TWIN1

# PYRAMIDAL_CA1_HLATEX7 - Latent hardening coupling coefficient - TWIN2

# PYRAMIDAL_CA2_TAU0X - Initial CRSS

# PYRAMIDAL_CA2_TAU1X - Back propagated CRSS - TAU0X

# PYRAMIDAL_CA2_THET0X - Initial hardening rate

# PYRAMIDAL_CA2_THET1X - Asymptotic hardening rate

# PYRAMIDAL_CA2_HPFACX - Hall-Petch contribution

# PYRAMIDAL_CA2_GNDFACX - GND hardening term

# PYRAMIDAL_CA2_HLATEX1 - Latent hardening coupling coefficient - PRISMATIC

# PYRAMIDAL_CA2_HLATEX2 - Latent hardening coupling coefficient - BASAL

# PYRAMIDAL_CA2_HLATEX3 - Latent hardening coupling coefficient - PYRAMIDAL_A

# PYRAMIDAL_CA2_HLATEX4 - Latent hardening coupling coefficient - PYRAMIDAL_CA1

# PYRAMIDAL_CA2_HLATEX5 - Latent hardening coupling coefficient - PYRAMIDAL_CA2

# PYRAMIDAL_CA2_HLATEX6 - Latent hardening coupling coefficient - TWIN1

# PYRAMIDAL_CA2_HLATEX7 - Latent hardening coupling coefficient - TWIN2

# TWIN1_TAU0X - Initial CRSS

# TWIN1_TAU1X - Back propagated CRSS - TAU0X

# TWIN1_THET0X - Initial hardening rate

# TWIN1_THET1X - Asymptotic hardening rate

# TWIN1_HPFACX - Hall-Petch contribution

# TWIN1_GNDFACX - GND hardening term

# TWIN1_HLATEX1 - Latent hardening coupling coefficient - PRISMATIC

# TWIN1_HLATEX2 - Latent hardening coupling coefficient - BASAL

# TWIN1_HLATEX3 - Latent hardening coupling coefficient - PYRAMIDAL_A

# TWIN1_HLATEX4 - Latent hardening coupling coefficient - PYRAMIDAL_CA1

# TWIN1_HLATEX5 - Latent hardening coupling coefficient - PYRAMIDAL_CA2

# TWIN1_HLATEX6 - Latent hardening coupling coefficient - TWIN1

# TWIN1_HLATEX7 - Latent hardening coupling coefficient - TWIN2

# TWIN2_TAU0X - Initial CRSS

# TWIN2_TAU1X - Back propagated CRSS - TAU0X

# TWIN2_THET0X - Initial hardening rate

# TWIN2_THET1X - Asymptotic hardening rate

# TWIN2_HPFACX - Hall-Petch contribution

# TWIN2_GNDFACX - GND hardening term

# TWIN2_HLATEX1 - Latent hardening coupling coefficient - PRISMATIC

# TWIN2_HLATEX2 - Latent hardening coupling coefficient - BASAL

# TWIN2_HLATEX3 - Latent hardening coupling coefficient - PYRAMIDAL_A

# TWIN2_HLATEX4 - Latent hardening coupling coefficient - PYRAMIDAL_CA1

# TWIN2_HLATEX5 - Latent hardening coupling coefficient - PYRAMIDAL_CA2

# TWIN2_HLATEX6 - Latent hardening coupling coefficient - TWIN1

# TWIN2_HLATEX7 - Latent hardening coupling coefficient - TWIN2

# <label>= <value>, <fix?>, <minval>, <maxval>

PRISMATIC_TAU0X= 80, 1,  0, 10000

PRISMATIC_TAU1X= 190, 1,  0, 10000

PRISMATIC_THET0X= 400, 1,  0, 10000

PRISMATIC_THET1X= 0, 1,  0, 10000

PRISMATIC_HPFACX= 0, 1,  0, 1e+06

PRISMATIC_GNDFACX= 0, 1,  0, 1e+06

PRISMATIC_HLATEX1= 1, 1,  0, 10000

PRISMATIC_HLATEX2= 0.5, 1,  0, 10000

PRISMATIC_HLATEX3= 0, 1,  0, 10000

PRISMATIC_HLATEX4= 0, 1,  0, 10000

PRISMATIC_HLATEX5= 1, 1,  0, 10000

PRISMATIC_HLATEX6= 2, 1,  0, 10000

PRISMATIC_HLATEX7= 0, 1,  0, 10000

BASAL_TAU0X= 25, 1,  0, 10000

BASAL_TAU1X= 30, 1,  0, 10000

BASAL_THET0X= 700, 1,  0, 10000

BASAL_THET1X= 0, 1,  0, 10000

BASAL_HPFACX= 0, 1,  0, 1e+06

BASAL_GNDFACX= 0, 1,  0, 1e+06

BASAL_HLATEX1= 2, 1,  0, 10000

BASAL_HLATEX2= 1, 1,  0, 10000

BASAL_HLATEX3= 0, 1,  0, 10000

BASAL_HLATEX4= 0, 1,  0, 10000

BASAL_HLATEX5= 5, 1,  0, 10000

BASAL_HLATEX6= 6, 1,  0, 10000

BASAL_HLATEX7= 0, 1,  0, 10000

PYRAMIDAL_A_TAU0X= 1500, 1,  0, 10000

PYRAMIDAL_A_TAU1X= 90, 1,  0, 10000

PYRAMIDAL_A_THET0X= 1000, 1,  0, 10000

PYRAMIDAL_A_THET1X= 50, 1,  0, 10000

PYRAMIDAL_A_HPFACX= 0, 1,  0, 1e+06

PYRAMIDAL_A_GNDFACX= 0, 1,  0, 1e+06

PYRAMIDAL_A_HLATEX1= 1, 1,  0, 10000

PYRAMIDAL_A_HLATEX2= 1, 1,  0, 10000

PYRAMIDAL_A_HLATEX3= 0, 1,  0, 10000

PYRAMIDAL_A_HLATEX4= 0, 1,  0, 10000

PYRAMIDAL_A_HLATEX5= 1, 1,  0, 10000

PYRAMIDAL_A_HLATEX6= 90, 1,  0, 10000

PYRAMIDAL_A_HLATEX7= 0, 1,  0, 10000

PYRAMIDAL_CA1_TAU0X= 320, 1,  0, 10000

PYRAMIDAL_CA1_TAU1X= 60, 1,  0, 10000

PYRAMIDAL_CA1_THET0X= 100, 1,  0, 10000

PYRAMIDAL_CA1_THET1X= 0, 1,  0, 10000

PYRAMIDAL_CA1_HPFACX= 100, 1,  0, 1e+06

PYRAMIDAL_CA1_GNDFACX= 300000, 1,  0, 1e+06

PYRAMIDAL_CA1_HLATEX1= 1, 1,  0, 10000

PYRAMIDAL_CA1_HLATEX2= 1, 1,  0, 10000

PYRAMIDAL_CA1_HLATEX3= 0, 1,  0, 10000

PYRAMIDAL_CA1_HLATEX4= 0, 1,  0, 10000

PYRAMIDAL_CA1_HLATEX5= 1, 1,  0, 10000

PYRAMIDAL_CA1_HLATEX6= 1, 1,  0, 10000

PYRAMIDAL_CA1_HLATEX7= 0, 1,  0, 10000

PYRAMIDAL_CA2_TAU0X= 60, 1,  0, 10000

PYRAMIDAL_CA2_TAU1X= 200, 1,  0, 10000

PYRAMIDAL_CA2_THET0X= 5000, 1,  0, 10000

PYRAMIDAL_CA2_THET1X= 1000, 1,  0, 10000

PYRAMIDAL_CA2_HPFACX= 0, 1,  0, 1e+06

PYRAMIDAL_CA2_GNDFACX= 0, 1,  0, 1e+06

PYRAMIDAL_CA2_HLATEX1= 1, 1,  0, 10000

PYRAMIDAL_CA2_HLATEX2= 1, 1,  0, 10000

PYRAMIDAL_CA2_HLATEX3= 0, 1,  0, 10000

PYRAMIDAL_CA2_HLATEX4= 0, 1,  0, 10000

PYRAMIDAL_CA2_HLATEX5= 1, 1,  0, 10000

PYRAMIDAL_CA2_HLATEX6= 3.5, 1,  0, 10000

PYRAMIDAL_CA2_HLATEX7= 0, 1,  0, 10000

TWIN1_TAU0X= 80, 1,  0, 10000

TWIN1_TAU1X= 0, 1,  0, 10000

TWIN1_THET0X= 0, 1,  0, 10000

TWIN1_THET1X= 0, 1,  0, 10000

TWIN1_HPFACX= 0, 1,  0, 1e+06

TWIN1_GNDFACX= 0, 1,  0, 1e+06

TWIN1_HLATEX1= 1, 1,  0, 10000

TWIN1_HLATEX2= 1, 1,  0, 10000

TWIN1_HLATEX3= 0, 1,  0, 10000

TWIN1_HLATEX4= 0, 1,  0, 10000

TWIN1_HLATEX5= 1, 1,  0, 10000

TWIN1_HLATEX6= 1, 1,  0, 10000

TWIN1_HLATEX7= 0, 1,  0, 10000

TWIN2_TAU0X= 270, 1,  0, 10000

TWIN2_TAU1X= 30, 1,  0, 10000

TWIN2_THET0X= 1000, 1,  0, 10000

TWIN2_THET1X= 178, 1,  0, 10000

TWIN2_HPFACX= 0, 1,  0, 1e+06

TWIN2_GNDFACX= 0, 1,  0, 1e+06

TWIN2_HLATEX1= 1, 1,  0, 10000

TWIN2_HLATEX2= 1, 1,  0, 10000

TWIN2_HLATEX3= 0, 1,  0, 10000

TWIN2_HLATEX4= 0, 1,  0, 10000

TWIN2_HLATEX5= 10, 1,  0, 10000

TWIN2_HLATEX6= 5, 1,  0, 10000

TWIN2_HLATEX7= 0, 1,  0, 10000